Superellipsoid-Constrained Bidirectional RRT: Path Planning with Adaptive AsymmetricDual-Super-Ellipsoidal Constrained Sampling

Yan Wu, Xingyu Guo1, Muchun Fan1, Feng Hong1, Hongyu Gao1, Jianyu Zheng1

Abstract: Sampling-based path planning algorithms commonly suffer from excessive invalid samples, slow convergence, and reduced success rates in high-obstacle-density environments. This paper presents SC-RRT, a path planning algorithm based on adaptive asymmetric dual-super-ellipsoid constrained sampling. Within a bidirectional parallel expansion framework, SC-RRT employs an intermediate meeting point xmeet to decompose the global task into staged subproblems. Scale-variable, asymmetric informed sampling subsets are constructed in both directions to concentrate sampling toward feasible regions with greater improvement potential, forming the Adaptive Dual-Super-Ellipsoid Constrained Sampling strategy (ADCS). To suppress invalid sampling caused by over-inflation or premature contraction of the informed subset due to cost estimation bias, a Search State Feedback Online Regulation mechanism (SSFOR) is developed. SSFOR uses the historical evolution of the optimal path cost to perform closed-loop adjustment of the super-ellipsoid expansion factor γ and heuristic sampling probability p, enabling the informed subset to evolve with search progress while reducing redundant expansion. Asymmetric super-ellipsoidal constraints further sharpen resolution along feasible passage directions. Simulation experiments show that SC-RRT reduces path length by approximately 10.5% compared with Dynamic-RRT, achieving a path efficiency of 0.9553. Although path quality is marginally lower than Informed RRT*, planning time is only 16.2% of the latter with node count reduced by 94.2%. SC-RRT thus balances path quality, planning efficiency, and computational cost without prior parameter tuning; its modular ADCS and SSFOR components can be independently integrated into other RRT-based frameworks.

Keywords: Sampling-based path planning, Informed sampling, bidirectional tree

search, online parameter regulation, motion planning

- Introduction

With the rapid advancement of Industry 4.0 and service robotics, intelligent robots have become a key driver of automation and operational efficiency across manufacturing, service, and related sectors[1,3]. Their application scope has expanded from structured factory settings to dynamic, complex environments where humans and robots work in close collaboration [4,5].Accordingly, in-depth research into robot path planning algorithms for complex environments carries not only considerable theoretical value, but also practical significance for advancing the real-world engineering deployment of intelligent robots. The core objective of path planning is to generate a collision-free trajectory for a robot within a known configuration space defined by a start point and a goal point, subject to kinematic constraints, while balancing two fundamental requirements: real-time responsiveness and path optimality [6,7]. Indynamic environments, randomly distributed obstacles and system uncertainties further compound the difficulty of planning. The shrinkage of the feasible region under sampling constraints and the resulting decline in tree growth efficiency are particularly pronounced [8], making the simultaneous achievement of fast convergence and high-quality paths a central challenge in current research [9,10].

The Rapidly Exploring Random Tree (RRT) algorithm and its bidirectional variants, such as Bidirectional RRT*, are classical sampling-based path planning method[11,12]. Owing to their probabilistic completeness and ease of implementation, they have been widely applied in complex environments [13,14]. However, the traditional Bidirectional RRT* algorithm still suffers from inherent limitations, such as strong randomness in the sampling process, low search efficiency, and insufficient geometric smoothness of the planned paths [15,16]. These limitations restrict its performance in scenarios involving large-scale environments, dense obstacles, or dynamically distributed obstacles [17,18]. Researchers have proposed various improvement strategies from the perspectives of sampling guidance, coordination of search strategies, and path structure optimization. A representative line of work enhances convergence efficiency by improving the bidirectional search framework and sampling guidance mechanisms, with particular emphasis on improving accessibility in difficult to-sample scenarios such as narrow passages [19]. ARRT-Connect [20], proposed by Li and Chen (2022), as well as ABQ-RRT* [21] based on similar principles, adaptively adjust the sampling strategy according to the scale of unexplored regions identified through dual-tree exploration. By incorporating the principal component analysis (PCA) approach introduced by Tahirovic and Ferizbegovic (2018) in RRT [22], these methods enhance the directional growth of trees within elongated feasible corridors, thereby accelerating exploration and connectivity in narrow environments.

However, such methods may still generate a large number of redundant sampling points in regions unrelated to the optimal path when operating in high-dimensional spaces or complex environments with multiple passages, thereby reducing overall planning efficiency. In particular, as the environment scale increases or obstacle density rises, fixed sampling bias strategies struggle to adapt to the varying demands of different search stages, leading to diminished guidance effectiveness of the informed subset. Informed RRT* proposed by Gammell et al. [23,24] restricts the sampling space to an informed subset defined by a super-ellipsoid with the start and goal states as its foci, thereby concentrating the sampling process on regions that can potentially improve the cost of the current best path. This work establishes the theoretical foundation for cost-guided constrained sampling. Building upon this foundation, Gammell et al. further proposed the BIT* algorithm [25,26], which unifies informed sampling and graph search into a heuristic search over an implicit random geometric graph, thereby further improving planning efficiency in high-dimensional scenarios. To further reduce the sensitivity of fixed super-ellipsoids to cost estimation bias, recent studies have attempted to introduce dynamic corridor constraints [27] or adaptive bidirectional super-ellipsoid sampling [28] to enhance the adaptability of the sampling domain. The Dynamic-RRT algorithm proposed by Zhao et al. innovatively introduces a heuristic super-ellipsoid constrained sampling mechanism. By estimating the path cost through expanded nodes, it restricts the sampling space to a super-ellipsoid with the start and goal states as its foci and the estimated path length as the major axis, thereby significantly improving the effectiveness of new node generation [29]. Meanwhile, the algorithm incorporates the Pareto Dominance principle [30] and dynamic programming concepts. By selecting Pareto-optimal nodes as new starting points to decompose the planning problem, it further reduces convergence time while ensuring path length optimization. Its performance is significantly superior to classical algorithms such as RRT, RRT*, and Informed RRT. Such methods effectively balance convergence speed and path quality through sampling space constraints and problem decomposition strategies. However, the performance of the sampling constraint mechanism in this algorithm still heavily depends on the accuracy of path cost estimation. When the cost is overestimated, the ellipsoidal constraint region becomes excessively large, leading to a considerable number of invalid samples; when the cost is underestimated, the constraint region shrinks prematurely, potentially excluding feasible optimal paths from the sampling space and causing the algorithm to converge to a suboptimal solution or even fail. Moreover, Dynamic-RRT adopts a unidirectional search framework, in which the informed subset is defined solely by the start and goal as focal points. In complex environments, its ability to guide the global search direction remains limited, and redundant expansions may still occur.

Overall, existing improvement methods still exhibit a pronounced trade-off between efficiency and optimality. While sampling guidance or bidirectional connectivity strategies can accelerate the acquisition of feasible solutions, they generally lack guarantees of asymptotic optimality. In contrast, cost-driven constrained sampling and problem decomposition approaches can increase the proportion of effective samples and improve path quality; however, their performance is highly sensitive to the accuracy of cost estimation. Moreover, the constraint domains are typically static or only weakly adaptive, which may lead to excessive shrinkage in the early search stage, reducing reachability, or cause oscillations in sampling ratios and tree expansion behavior when cost values fluctuate, thereby weakening global robustness and convergence consistency. To this end, this paper proposes an SC-RRT algorithm based on an adaptive asymmetric dual-super-ellipsoid constrained sampling subset. Within a bidirectional parallel search framework guided by intermediate points, the algorithm constructs asymmetric dual-super-ellipsoid sampling subsets on the start and goal sides that can independently expand and contract. To overcome the sensitivity of traditional constrained sampling to cost estimation bias and to prevent premature shrinkage or ineffective expansion of the sampling space, a search state feedback-based online regulation mechanism is established. Using the current best path cost and its historical evolution as feedback, a closed-loop control strategy is employed to adjust the scale of the super-ellipsoid informed subsets. During the stage when a stable feasible solution has not yet been obtained, the constraints are moderately relaxed to enhance exploration coverage; once feasible paths emerge and continue to improve, the constraints are progressively strengthened to increase effective sampling density and suppress redundant expansions. In this way, a controllable dynamic balance is achieved among convergence speed, path quality, and search stability. Comparative experiments are conducted in representative scenarios with varying obstacle densities and spatial dimensions, using Dynamic-RRT and classical RRT variants as baselines. The effectiveness of the proposed mechanisms is systematically evaluated in terms of path length, planning time, node scale, and path smoothness.

- Proposed Method

  - Problem Formulation and Environment Modeling

To establish the mathematical model of the path planning problem [31], the Cartesian workspace W ⊆ R m and the configuration space C ⊆ Rn are defined, where m and n denote the corresponding dimensions, respectively [32]. Let the state be defined as:

and partition the space into:

• Collision-free region: Wfree (white area)

• Obstacle region: Wobs (black circular obstacles)

Such that:

The planning start and goal states are denoted by xstart and xgoal, respectively, where both satisfy xstart, xgoal ∈ Wfree, as illustrated in Figure 1. The SC-RRT algorithm constructs two random trees TA and TB from the start state xstart and the goal state xgoal, respectively, and introduces a dynamic intermediate point xmeet, thereby decoupling the global search into two subproblems: “start–intermediate” and “goal–intermediate,” as defined in Equation (1). The intermediate point xmeet is updated online based on the bidirectional search states. Unlike a traditional single static ellipsoid, the super-ellipsoid constructed in this paper does not maintain a fixed shape. Its parameters are dynamically updated as the path cost improves and the intermediate point shifts. Using xmeet as a bridge, the superellipsoid adaptively expands and contracts according to changes in path cost and the exploration states during the search process. Sampling constraints are constructed separately on the start side and the goal side, ensuring coverage of potentially optimal paths while progressively tightening the sampling domain boundaries, thereby effectively reducing invalid samples.

Fig.1 Schematic Diagram of the Search State-Driven Adaptive Asymmetric dual-super-ellipsoid

Constrained Sampling SC-RRT Path Planning Algorithm

  - An Adaptive Asymmetric Dual-Super-Ellipsoid Constrained Sampling (ADCS) Strategy

This paper proposes an asymmetric dual-super-ellipsoidal constrained sampling subset mechanism, as illustrated in Figure 2, to achieve asymmetric regulation of the sampling domain. Unlike the Probabilistic Roadmap (PRM) method [33], which adopts an offline planning paradigm based on a multi-query graph structure, the proposed approach balances global exploration and local refined search during bidirectional tree expansion. Ultimately, it achieves simultaneous reductions in path length, planning time, and the number of nodes.

The core of this sampling strategy lies in dynamically computing the constraint parameters of the super-ellipsoid. For either direction, taking the start side as an example, it is defined as follows:

Where:

•  denotes the expansion coefficient adaptively adjusted online by the SSFOR mechanism;

•  denotes the estimated optimal total cost from the start side based on heuristic evaluation;

• ∥x − f1∥ denotes the accumulated path cost from the start node xstart to node x;

• ∥x − f2∥ denotes the heuristic distance from node x to the intermediate point xmeet.

Fig. 2 Flowchart of the Adaptive Asymmetric dual-super-ellipsoid Constrained Sampling Strategy

The goal-side super-ellipsoid is defined analogously. The two super-ellipsoids are updated independently, forming an asymmetric constraint structure. In each iteration, informed sampling is conducted within the above super-ellipsoids with probability p; with a certain probability, biased sampling is performed toward the optimal node selected from the opposite tree according to the Pareto dominance principle; and with the remaining probability, global uniform sampling is carried out. This ensures that the search remains focused on effective regions while preserving global coverage capability.

For each newly expanded node, a direct connection to the opposite tree is attempted first. If the distance exceeds the threshold, the opposite tree greedily advances toward it. Once the two trees are successfully connected, node rewiring optimization and adaptive smoothing are performed on the concatenated path [34, 35],

yielding the final feasible path.

  - Search State Feedback-Based Online Regulation (SSFOR)Mechanism

The effectiveness of ADCS depends on the timely adjustment of the expansion coefficient γ and the informed sampling ratio p. Existing convergence-acceleration methods, such as path-biased sampling [36], batch forward-propagation search [37], cost lower bound trade-off strategies [38], and anytime planning frameworks [39, 40], generally adopt preset static parameters. Heuristic sampling pruning methods [41] also require predefined thresholds, and thus lack online feedback regulation based on the search state. To address this limitation, this paper proposes the SSFOR mechanism to implement closed-loop regulation of both parameters, as detailed in the flowchart of Figure 3. The relative rate of decrease in the estimated total optimal path cost is used as a signal of the current search progress. When the path cost shows no significant improvement over time, the constraints are considered potentially too restrictive or the search is deemed to have stagnated. Accordingly, γ is increased to expand the super-ellipsoidal coverage, and p is reduced to enhance global random exploration. Conversely, when the path cost continues to improve effectively, γ is decreased to tighten the constrained sampling subset, while p is increased to strengthen informed sampling and accelerate local convergence.

Fig. 3 Flow diagram of the Search State Feedback-Based Online Regulation (SSFOR) mechanism

To ensure the boundedness and reproducibility of the above adjustment process, this paper specifies the value range and initial states of the two parameters as follows: The super-ellipsoid expansion factor γ is defined within the range γ ∈ [1.0, 4.0], with a nominal value of γ0 = 1.5. At algorithm initialization, γ is set to 2.5 to ensure sufficient exploratory coverage during the early stage of the search. A larger γ corresponds to a greater degree of super-ellipsoidal expansion, leading the algorithm to favor broader exploration. Conversely, a smaller γ results in a more compact constrained sampling subset, causing the algorithm to shift toward intensified local convergence. The heuristic sampling probability p is defined within the range p ∈ [0.2, 0.95], with a nominal value of p0 = 0.8. The initial value is set to the lower bound p = 0.2, so that before any feasible path is obtained, the algorithm relies entirely on global random sampling for initial exploration. A higher p corresponds to a greater proportion of informed sampling, favoring refined local search; a lower p increases the proportion of global random sampling, thereby maintaining search diversity. When the constraint is overly relaxed, a large number of nodes fall into inefficient regions; when it is overly restrictive, the effective sampling rate drops sharply in obstacle-dense environments. Under the closed-loop regulation of SSFOR, both parameters fluctuate within the aforementioned valid ranges, preventing unbounded expansion or excessive contraction of the constrained sampling subset, and ensuring that the constraint strength continuously matches the current stage of the search. The overall framework of SC-RRT, comprising the main search loop, SSFOR-based online regulation, and ADCS sampling, is illustrated in Figure 4.

Fig. 4 Overall framework of the SC-RRT algorithm. It consists of the main search loop, SSFOR-based online regulation, and ADCS sampling. Dashed arrows indicate the internal implementation of

key modules

The sequential search process of SC-RRT in a representative 2D environment is depicted in Figure 5, where the progressive convergence of the dual-super-ellipsoid constrained sampling regions can be observed.

Fig. 5 Path planning process of SC-RRT in a 2D environment.

- Experiment

  - Algorithm Comparison Experiments in Baseline Environments基准环境下的算法对比实验

To systematically evaluate the performance of SC-RRT under consistent environmental scale and obstacle density, comparative simulation experiments were conducted in both two-dimensional and three-dimensional spaces. The results are shown in Figures 6 and 7. The evaluation metrics include: Path Length, defined as the sum of the Euclidean distances between adjacent nodes along the path (mm); Convergence Time, defined as the time required for the algorithm to achieve path convergence (s); Node Count, defined as the total number of sampled nodes generated during the planning process; and The path efficiency η is formally defined as:

Whereis the total path length along the planned path , and ∥xgoal − xstart∥ is the straight-line distance. Values of η closer to 1.0 indicate near-optimal paths.

Based on previous studies in path planning, five representative algorithms—RRT, RRT-Connect, RRT*, Informed RRT*, and Dynamic-RRT—were selected for comparison with the proposed SC-RRT algorithm to validate its performance and stability. To ensure fairness in performance comparison among different path planning algorithms, all methods were evaluated under identical environmental spaces and obstacle configurations. Using the environment specified in Table 1 as an example, each algorithm was executed 500 times, and performance metrics such as path length and convergence time were assessed.

Tab.1 Environment Configuration

| Dimension | Environment | Obstacles | Start | Goal | Max Iter |
| --- | --- | --- | --- | --- | --- |
| 2 | 1500*1500 | 225 | （75，75） | (1425，1425) | 5000 |
| 3 | 1500*1500*1500 | 400 | (75, 75, 75) | (1425，1425，1425) | 5000 |

Under typical scenarios with varying obstacle densities in both two-dimensional and three-dimensional spaces, a systematic comparison was conducted between SC-RRT and mainstream algorithms including RRT, RRT-Connect, RRT*, Informed RRT*, and Dynamic-RRT, under identical environmental configurations and computational budgets. The results are presented in Table 2. Overall, the asymptotic optimality of Dynamic-RRT relies on sufficient computation time to be fully realized, which leads to notable limitations in scenarios with high real-time requirements. Although biasing the probability distribution of the random sample node xrand toward the goal direction yields some improvement in path length and efficiency metrics, the degree of enhancement remains limited. In contrast, leveraging the online adaptive regulation mechanism (SSFOR), SC-RRT significantly reduces convergence time and sampling scale while maintaining comparable path length, demonstrating superior overall real-time performance compared to Dynamic-RRT. Regarding path quality, SC-RRT achieves an average path length of 2049.09 mm. This represents a 10.5% reduction compared to Dynamic-RRT and deviates by only 4.0% from the asymptotically optimal RRT*, yet it incurs a fraction of the computational cost. With a path efficiency of 0.9553, SC-RRT ranks third, trailing only the computationally intensive Informed RRT* and RRT*. Crucially, SC-RRT accomplishes this with a planning time that is just 16.2% of Informed RRT*’s requirement. The marginal 1.40% efficiency gap compared to RRT* highlights SC-RRT’s ability

to approach optimality without the associated time penalty. These metrics validate that the ADCS mechanism effectively concentrates sampling along near-straight corridors, minimizing detours while drastically accelerating convergence. Furthermore, the low standard deviation in path length (±57.53 mm) confirms the algorithm’s stability across diverse obstacle configurations. In terms of computational efficiency, SC-RRT achieves an average convergence time of 0.0345 s, with a node consumption of only 209.6, demonstrating excellent real-time responsiveness. Although RRT* guarantees asymptotic optimality, it requires as many as 3846.6 nodes and a convergence time of 0.1629 s. In comparison, SC-RRT achieves nearly a 4.7-fold speedup, highlighting its significant advantage in time-sensitive tasks.

Fig.6 Comparison Results of Different Path Planning Algorithms in Two-Dimensional Space.

In terms of path stability, SC-RRT exhibits a path length standard deviation of only ±57.53 mm, which is significantly lower than that of RRT (±126.25 mm) and Dynamic-RRT (±136.02 mm), demonstrating markedly superior consistency and reproducibility in planning results. In summary, SC-RRT achieves an optimal overall balance among path quality, computational efficiency, and planning stability. It maintains stable convergence and strong scalability across both two-dimensional and three-dimensional scenarios with varying obstacle densities, demonstrating robust generalization capability and substantial practical engineering value.

Fig.7 Comparison Results of Different Path Planning Algorithms in Three-Dimensional Space.

Tab.2 Comparison of Path Planning Algorithms

| Algorithm | Path Length (mm) | Convergence Time (s) | Node Count | Path Efficiency |
| --- | --- | --- | --- | --- |
| RRT | 2329.5500 ± 126.2500 | 0.0409 ± 0.0305 | 279.4000 ± 175.5000 | 0.8219 ± 0.0429 |
| RRT-connect | 2143.2400 ± 93.4900 | 0.0108 ± 0.0053 | 69.3000 ± 10.8000 | 0.8924 ± 0.0376 |
| RRT* | 1970.7200 ± 19.9400 | 0.1629 ± 0.0843 | 3846.6000 ± 99.7000 | 0.9689 ± 0.0098 |
| Informed RRT* | 1938.4500 ± 24.1300 | 0.2134 ± 0.0487 | 3612.3000 ± 108.6000 | 0.9714 ± 0.0076 |
| SC-RRT | 2049.0900 ± 57.5300 | 0.0345 ± 0.0124 | 209.6000 ± 38.5000 | 0.9553 ± 0.0265 |
| Dynamic-RRT | 2288.1300 ± 136.0200 | 0.0414 ± 0.0384 | 281.3000 ± 160.2000 | 0.8372 ± 0.0468 |

  - Real-world Validation of SC-RRT on a Robotic Manipulation Platform

为进一步验证SC-RRT算法由仿真环境向真实机器人系统迁移的有效性，本文在实际机器人操作平台上开展了实验验证。该实验并非针对具体应用任务的系统性能进行评价，而是选取具有复杂三维曲面结构和连续空间运动需求的实际任务场景，对所提出路径规划算法在真实环境中的轨迹生成能力和执行稳定性进行验证，表3为机械臂的实验平台具体配置信息，其中，一维力传感器仅用于监测机器人执行过程中的末端接触状态，并未参与路径规划过程或轨迹优化。因此，本文不将力相关指标作为SC-RRT性能评价指标。。

实验过程中，人体曲面区域被作为典型非结构化三维工作空间，多个任务关键点被定义为连续路径规划目标点。该场景具有空间约束复杂、目标点分布连续以及末端姿态变化明显等特点，与SC-RRT算法中双向搜索和超椭球约束采样机制所面向的复杂空间规划问题具有一致性。因此，该实验能够有效评价SC-RRT算法在真实机器人环境中的适应能力。真实机器人实验流程如图8所示。由深度视觉模块获取目标工作空间的RGB-D信息。通过深度相机完成深度数据解析与点云重建，获得环境三维几何信息。在视觉识别结果提取多个空间目标点，并将其作为连续轨迹规划任务中的关键节点。针对相邻目标点之间存在的复杂空间约束，SC-RRT规划器利用双向搜索策略缩小搜索区域，并通过超椭球约束采样机制提高有效采样比例，实现多个目标点之间的连续无碰轨迹生成。最终，规划得到的笛卡尔空间轨迹被转换为机器人运动指令，由机械臂执行实际运动。

Tab.3 Configuration of the Real Robotic Experimental Platform（表格作用是机械臂实验平台配置）

| Component | Specification |
| --- | --- |
| Robot manipulator | AUBO-i10 |
| Degrees of freedom | 6-DOF |
| End-effector sensing | KWT1D60-300 |
| Perception system | Deptrum Aurora 930 RGB-D camera |
| Planning algorithm | SC-RRT |

图8展示了SC-RRT算法在真实机器人平台中的实验验证场景。实验采用AUBO-i10六自由度机械臂作为运动执行机构，并结合RGB-D视觉系统获取目标工作区域的三维结构信息。图8(a)展示了整体实验环境，机械臂根据视觉感知结果在真实人体曲面环境中执行空间运动任务。图8(b)(c)为机器人末端执行器执行状态，可以观察到末端能够按照规划轨迹保持稳定运动，验证了所生成轨迹在真实机器人系统中的可执行性。

与理想化仿真环境不同，真实实验过程中存在视觉测量误差、环境几何不确定性以及机器人执行误差等因素。因此，该实验主要用于评价SC-RRT算法在真实三维非结构化环境中的轨迹生成能力和运动稳定性。

图 8(a) 末端执行器贴合细节  图 8(b) 肩胛区域作业姿态

图 8(c) 系统全景

Figure 8: SC-RRT algorithm execution on real human back

图9展示了基于RGB-D视觉感知的三维环境建模过程以及SC-RRT规划过程。通过Deptrum Aurora 930 RGB-D相机采集目标区域的彩色与深度信息，并利用ROS2驱动节点完成RGB-D数据同步、坐标转换以及点云重建，获得人体背部曲面的三维空间表示。

在完成环境建模后，根据视觉识别结果提取多个空间目标点，并将其作为连续路径规划任务中的关键节点。如图9所示，SC-RRT算法在重建点云环境中进行搜索，并利用双向扩展策略和超椭球约束采样机制限制无效搜索区域，提高复杂空间中的路径搜索效率。

图 9(a)机械臂与人体背部点云整体场景  图 9(b)末端执行器的姿态约束特写

图 9(c) SC-RRT算法路径贴合人体表面特写

Fig.9 Visualization of the robotic massage experiment in a real-world setting.

图10展示了SC-RRT算法在真实三维点云环境中生成的最终运动轨迹。针对多个空间目标点之间的连续运动需求，SC-RRT通过约束采样策略在复杂曲面环境中建立有效搜索空间，并生成连续、平滑的机器人运动路径。从图10可以看出，规划轨迹能够覆盖多个目标位置，并保持与环境曲面相适应的空间运动趋势。该结果表明，所提出方法能够有效处理真实场景中的多目标路径规划问题，并实现从视觉感知环境到机器人运动执行的有效迁移。

Fig.10 机械臂执行轨迹

Tab.4 真实机械臂实验性能指标 (Performance Metrics of Real-world Robotic Massage Experiment)

| Metric（指标） | Value（数值） |
| --- | --- |
| Planning time | 0.037ms |
| Executed path length | 3.245m |
| Planned path length | 3.249m |
| Path deviation | 0.14% |
| TCP tracking error | 1.98mm(mean),3.09mm(max) |
| Maximum joint variation | 29.78° |
| High-frequency joint fluctuation | 0.047° |

结果表明，SC-RRT 生成轨迹的平均规划计算时间为 3.052s，满足实时机器人运动规划需求，规划轨迹长度为3.249 m，实际执行轨迹长度为3.245 m，两者偏差仅为0.14%，表明规划结果能够被机器人系统稳定执行。此外，末端执行器跟踪误差均值为1.98 mm，最大误差为3.09 mm，说明机器人能够较准确地跟踪SC-RRT生成的运动轨迹。在运动过程中，关节角变化范围最大为29.78°，高频运动残差控制在0.047°以内，未出现明显振荡现象。上述结果表明，SC-RRT生成的轨迹能够被真实机器人系统稳定执行，在复杂三维曲面环境下保持较高的轨迹跟踪精度。这验证了所提出方法不仅能够在仿真环境中提升搜索效率，同时能够适应真实机器人系统中的环境感知误差和运动执行误差。

- Conclusions

To address the issues of low effective sampling rates, slow convergence in obstacle dense environments, and the sensitivity of informed subsets to cost estimation bias in sampling based path planning algorithms, this paper proposes the SC-RRT algorithm.The algorithm comprises two tightly coupled core mechanisms.

- Adaptive Asymmetric Dual-Super-Ellipsoidal Constrained Sampling (ADCS). A dynamically updated intermediate point, derived online from the bidirectional search state, is introduced to decompose the global planning task into two directional subproblems: “start–intermediate” and “goal–intermediate.” For each direction, a super-ellipsoidal informed subset with independently adjustable scale is constructed. Unlike Dynamic-RRT, which adopts a single symmetric ellipsoid with the start and goal as fixed focal points, the asymmetric dual-super-ellipsoid structure of ADCS enables directional compression and expansion according to the differing search progress on the two sides. While maintaining exploratory coverage, it significantly increases the sampling density along potentially feasible corridors, effectively mitigating the insufficient global guidance capability of single-direction frameworks highlighted in the introduction.

- The Search-State Feedback Online Regulation (SSFOR) mechanism. Using the current best path cost and its recent historical sequence as feedback signals, SSFOR dynamically outputs the super-ellipsoid expansion factor γ and the informed sampling probability p, implementing closed-loop regulation over the volume of the informed subset and the sampling allocation ratio. When path improvement stagnates, the exploration range is automatically expanded; when improvement is efficient, the constraints are progressively tightened to enhance local convergence. In this way, the mechanism overcomes the sensitivity issues in traditional methods, where cost overestimation leads to an excessively large informed subset and cost underestimation resultsin premature contraction of the sampling region.

Under representative scenarios with varying obstacle densities in both twodimensional and three-dimensional spaces, and under identical environmental configurations and computational budgets, SC-RRT—through the synergistic effect of ADCS’s bidirectional asymmetric constraints and SSFOR’s closed-loop regulation demonstrates more consistent convergence behavior, fewer redundant expansions, and higher planning stability across different spatial dimensions and obstacle distributions. These results confirm that the integration of adaptive asymmetric dual-superellipsoidal constrained sampling with online constraint feedback regulation is an effective approach to enhancing the overall performance of sampling-based planning algorithms. It further provides a generalizable modeling framework for mitigating sensitivity to cost estimation bias in similar methods. SC-RRT inherits the probabilistic completeness of Informed RRT* and, within the asymptotic optimality framework, further accelerates convergence through bidirectional expansion and dynamic constraint regulation, thereby ensuring both theoretical soundness and engineering efficiency.

Acknowledgements

The authors would like to thank the School of Mechanical Engineering, Shanghai Institute of Technology, for providing the computational

resources.

AI disclosure

During the preparation of this manuscript, the authors used GitHub Copilot as a translation aid to translate the original Chinese draft into English. All AI-assisted output was carefully reviewed and edited by the authors, who take full responsibility for the accuracy, scientific integrity, and originality of the manuscript.

Declarations

Author contributions: Xingyu Guo conceived the algorithm, conducted the simulation experiments, and drafted the manuscript. Yan Wu supervised the research and provided guidance on the overall manuscript. Muchun Fan and Feng Hong provided technical support during implementation. Hongyu Gao and Jianyu Zheng contributed to manuscript revision and provided valuable comments. All authors contributed to the conception and design of the study, and all authors read and approved the final manuscript.

Funding

This research received no specific grant from any funding agency, commercial, or not-for-profit sectors.

Competing interests

The authors declare no competing interests.

Ethics approval

Not applicable.

Consent to participate

Not applicable.

Consent for publication

All authors have approved the manuscript and agree to its submission for publication.

Data availability

The data that support the findings of this study are available from the corresponding author upon reasonable request.

Code availability

The code used in this study is available from the corresponding author upon reasonable request.

References

- Zhong, R.Y., Xu, X., Klotz, E., Newman, S.T.: Intelligent manufacturing in the context of industry 4.0: a review. Engineering 3(5), 616–630 (2017)

- Wirtz, J., Patterson, P.G., Kunz, W.H., Gruber, T., Lu, V.N., Paluch, S., Martins, A.: Brave new world: service robots in the frontline. Journal of service management 29(5), 907–931 (2018)

- Mac, T.T., Copot, C., Tran, D.T., De Keyser, R.: Heuristic approaches in robot path planning: A survey. Robotics and Autonomous Systems 86, 13–28 (2016)

- Ajoudani, A., Zanchettin, A.M., Ivaldi, S., Albu-Sch¨affer, A., Kosuge, K., Khatib, O.: Progress and prospects of the human–robot collaboration. Autonomous robots 42(5), 957–975 (2018)

- Villani, V., Pini, F., Leali, F., Secchi, C.: Survey on human–robot collaboration in industrial settings: Safety, intuitive interfaces and applications. Mechatronics 55, 248–266 (2018)

- LaValle, S.M.: Planning Algorithms. Cambridge university press, Cambridge (2006)

- Latombe, J.-C.: Robot Motion Planning vol. 124. Springer, New York (2012)

- Kingston, Z., Moll, M., Kavraki, L.E.: Sampling-based methods for motion planning with constraints. Annual review of control, robotics, and autonomous systems 1(1), 159–185 (2018)

- Mohanan, M., Salgoankar, A.: A survey of robotic motion planning in dynamic environments. Robotics and Autonomous Systems 100, 171–185 (2018)

- Aggarwal, S., Kumar, N.: Path planning techniques for unmanned aerial vehicles: A review, solutions, and challenges. Computer communications 149, 270–299 (2020)

- Elbanhawi, M., Simic, M.: Sampling-based robot motion planning: A review. Ieee access 2, 56–77 (2014)

- Karaman, S., Frazzoli, E.: Sampling-based algorithms for optimal motion planning. The international journal of robotics research 30(7), 846–894 (2011)

- LaValle, S.: Rapidly-exploring random trees: A new tool for path planning. Research Report 9811 (1998)

- LaValle, S.M., Kuffner Jr, J.J.: Randomized kinodynamic planning. The international journal of robotics research 20(5), 378–400 (2001)

- Kuffner, J.J., LaValle, S.M.: Rrt-connect: An efficient approach to single-query path planning. In: Proceedings 2000 ICRA. Millennium Conference. IEEE International Conference on Robotics and Automation. Symposia Proceedings (Cat.No. 00CH37065), vol. 2, pp. 995–1001 (2000). IEEE

- Gammell, J.D., Strub, M.P.: Asymptotically optimal sampling-based motion planning methods. Annual Review of Control, Robotics, and Autonomous Systems 4(1), 295–318 (2021)

- Qureshi, A.H., Ayaz, Y.: Intelligent bidirectional rapidly-exploring random trees for optimal motion planning in complex cluttered environments. Robotics and Autonomous Systems 68, 1–11 (2015)

- Ding, J., Zhou, Y., Huang, X., Song, K., Lu, S., Wang, L.: An improved rrt* algorithm for robot path planning based on path expansion heuristic sampling. Journal of computational science 67, 101937 (2023)

- Noreen, I., Khan, A., Habib, Z., et al.: Optimal path planning using rrt* based approaches: a survey and future directions. International Journal of Advanced Computer Science and Applications 7(11), 97–107 (2016)

- Li, B., Chen, B.: An adaptive rapidly-exploring random tree. IEEE/CAA Journal of Automatica Sinica 9(2), 283–294 (2021)

- Huang, Z., Gao, Y., Guo, J., Qian, C., Chen, Q.: An adaptive bidirectional quick optimal rapidly-exploring random tree algorithm for path planning. Engineering Applications of Artificial Intelligence 135, 108776 (2024)

- Tahirovic, A., Ferizbegovic, M.: Rapidly-exploring random vines (rrv) for motion planning in configuration spaces with narrow passages. In: 2018 IEEE International Conference on Robotics and Automation (ICRA), pp. 7055–7062 (2018). IEEE

- Gammell, J.D., Srinivasa, S.S., Barfoot, T.D.: Informed rrt*: Optimal samplingbased path planning focused via direct sampling of an admissible ellipsoidal heuristic. In: 2014 IEEE/RSJ International Conference on Intelligent Robots and Systems, pp. 2997–3004 (2014). IEEE

- Gammell, J.D., Barfoot, T.D., Srinivasa, S.S.: Informed sampling for asymptotically optimal path planning. IEEE Transactions on Robotics 34(4), 966–984 (2018)

- Gammell, J.D., Srinivasa, S.S., Barfoot, T.D.: Batch informed trees (bit*): Sampling-based optimal planning via the heuristically guided search of implicit random geometric graphs. In: 2015 IEEE International Conference on Robotics and Automation (ICRA), pp. 3067–3074 (2015). IEEE

- Gammell, J.D., Barfoot, T.D., Srinivasa, S.S.: Batch informed trees (bit*): Informed asymptotically optimal anytime search. The International Journal of Robotics Research 39(5), 543–567 (2020)

- Yuan, L., Zhao, J., Li, W., Hou, J.: Improved informed-rrt* based path planning and trajectory optimization for mobile robots. International Journal of Precision Engineering and Manufacturing 24(3), 435–446 (2023)

- Zeng, Z., Chang, Y.: Abf-rrt*: Bidirectional fast optimal rapidly-exploring random tree via adaptive sampling of a permissible hyper-ellipsoid. Journal of King Saud University Computer and Information Sciences 37(7), 165 (2025)

- Zhao, P., Chang, Y., Wu, W., Luo, H., Zhou, Z., Qiao, Y., Li, Y., Zhao, C., Huang, Z., Liu, B., et al.: Dynamic rrt: fast feasible path planning in randomly distributed obstacle environments. Journal of Intelligent & Robotic Systems 107(4), 48 (2023)

- Deb, K., Pratap, A., Agarwal, S., Meyarivan, T.: A fast and elitist multiobjective genetic algorithm: Nsga-ii. IEEE transactions on evolutionary computation 6(2), 182–197 (2002)

- Choset, H., Lynch, K.M., Hutchinson, S., Kantor, G.A., Burgard, W.: Principles of Robot Motion: Theory, Algorithms, and Implementations. MIT press, Boston (2005)

- Lynch, K.M., Park, F.C.: Modern Robotics. Cambridge University Press, Cambridge (2017)

- Kavraki, L.E., Svestka, P., Latombe, J.-C., Overmars, M.H.: Probabilistic roadmaps for path planning in high-dimensional configuration spaces. IEEE transactions on Robotics and Automation 12(4), 566–580 (2002)

- Ravankar, A., Ravankar, A.A., Kobayashi, Y., Hoshino, Y., Peng, C.-C.: Path smoothing techniques in robot navigation: State-of-the-art, current and future challenges. Sensors 18(9), 3170 (2018)

- Yang, K., Sukkarieh, S.: An analytical continuous-curvature path-smoothing algorithm. IEEE Transactions on Robotics 26(3), 561–568 (2010)

- Nasir, J., Islam, F., Malik, U., Ayaz, Y., Hasan, O., Khan, M., Muhammad, M.S.: Rrt*-smart: A rapid convergence implementation of rrt. International Journal of Advanced Robotic Systems 10(7), 299 (2013)

- Janson, L., Schmerling, E., Clark, A., Pavone, M.: Fast marching tree: A fast marching sampling-based method for optimal motion planning in many dimensions. The International journal of robotics research 34(7), 883–921 (2015)

- Salzman, O., Halperin, D.: Asymptotically near-optimal rrt for fast, high-quality motion planning. IEEE Transactions on Robotics 32(3), 473–483 (2016)

- Ferguson, D., Stentz, A.: Anytime rrts. In: 2006 IEEE/RSJ International Conference on Intelligent Robots and Systems, pp. 5369–5375 (2006). IEEE

- Karaman, S., Walter, M.R., Perez, A., Frazzoli, E., Teller, S.: Anytime motion planning using the rrt. In: 2011 IEEE International Conference on Robotics and Automation, pp. 1478–1483 (2011). ieee

- Akgun, B., Stilman, M.: Sampling heuristics for optimal motion planning in high dimensions. In: 2011 IEEE/RSJ International Conference on Intelligent Robots and Systems, pp. 2640–2645 (2011). IEEE

Statements & Declarations

Funding

The authors declare that no funds, grants, or other support were received during the preparation of this manuscript.

Competing Interests

The authors have no relevant financial or non-financial interests to disclose.

Author Contributions

All authors contributed to the study's conceptual design. Yan Wu supervised the entire manuscript writing process; Xingyu Guo, Wenjun Lv, and Jianyu Zheng prepared the materials; Jiaqing Ye and Jiale Hu collected and analyzed the experimental data; and Xingyu Guo drafted the manuscript. All authors provided revision suggestions, participated in revising all versions, and finally reviewed and approved the final draft.

School of Mechanical Engineering, Shanghai Institute of Technology, Shanghai 201418, China

Corresponding Author:Xingyu Guo

E-mail: gxyzhendehenlihai@163.com