SC-RRT: Bidirectional Adaptive Superellipsoid Constrained Sampling for Path Planning in Cluttered Obstacle Environments

基于自适应非对称双椭球域约束采样的SC-RRT算法

Xingyu Guo1 · YanWu2 · HongYu Gao2 · JiaLe Hu1 · FengHong1 · MuChun Fan1

Abstract:

针对复杂障碍环境中采样式路径规划算法易产生无效采样、收敛速度慢且在高障碍密度下成功率下降的问题，本文提出了一种基于自适应非对称双椭球约束采样的SC-RRT路径规划算法。该方法在双向并行扩展框架下引入中介点引导，将全局规划任务分解为阶段性子问题，通过构造尺度可变且允许非对称伸缩的双向知情采样域，使采样逐步聚焦于更具改进潜力的可行路径，形成自适应双椭球约束采样策略（Adaptive Dual-Ellipsoid Constrained Sampling-ADCS），在维持探索覆盖性的同时显著提高有效采样密度。为降低代价估计偏差引起的知情子集过度膨胀或过早收缩导致的无效采样，本文建立了基于搜索状态反馈的在线调节机制（Search State Feedback-Based Online Regulation），利用当前最优路径代价的历史演化为反馈，对超椭球扩张因子γ与启发式采样概率p进行闭环调节，从而构建随搜索进程实时演化的知情采样子集，使采样密度在全局探索空间与局部约束区域间自适应分配，有效抑制了冗余扩展并维持了搜索的连续性，结合非对称超椭球体约束策略对采样空间进行方向性压缩与扩展，增强了算法在潜在可行通道方向上的分辨能力。

仿真实验在不同障碍密度与空间维度的测试场景中验证了算法性能：在与对比方法相同的环境配置与计算预算约束下，SC-RRT 相比 Dynamic-RRT 平均路径长度降低约 15%–20%，路径平滑度提升约79%，相比RRT*算法，SC-RRT在获得更优路径质量的同时规划耗时缩短超过两个数量级。实验结果表明，SC-RRT能够在保持搜索完整性的同时有效抑制无效探索，使搜索树具备更稳定的扩展行为与更强的环境适应能力。

补充研究意义作为总结

Keywords： 快速扩展随机树（RRT）、知情采样、非对称双向超椭球体、在线调节机制、路径规划

- Introduction

随着工业4.0与服务机器人技术的迅猛发展，智能机器人已成为提升制造、服务等领域自动化水平与作业效率的核心载体，其应用场景已从结构化工厂延伸至动态复杂的人机协作环境[1–3]。这一趋势对机器人的自主决策能力提出了更高要求，尤其是作为核心技术模块的路径规划算法，其性能直接决定机器人作业的安全性、可靠性与实时性[4–6]。因此，针对复杂环境下的机器人路径规划算法展开深入研究，不仅具有重要的理论价值，更对推动智能机器人的工程化应用具有关键意义[7, 8]。路径规划的核心任务是在已知起点与目标点的配置空间中，为机器人生成一条满足运动约束的无碰撞轨迹，同时平衡“实时响应”与“路径最优”两大核心目标[9]。在动态环境下，随机分布的障碍物及系统不确定性进一步加剧了规划难度，使得同时实现快速收敛与高质量路径成为当前研究中的关键挑战[10–12]。

快速探索随机树（Rapidly Exploring Random Tree, RRT）算法及其双向扩展形式如双向RRT*是一类经典的采样式路径规划方法，凭借其概率完备性与易实现性，在复杂环境中得到了广泛应用 [7, 8]。然而，传统双向 RRT * 算法仍存在固有缺陷，如采样过程随机性强、搜索效率低下以及规划路径几何不够平滑等问题 [13]。这些不足限制了其在环境尺度大、障碍物密集或动态障碍物分布场景中的性能表现。为解决上述问题，近年来研究者从采样引导、搜索策略协同以及路径结构优化等角度提出了多种改进方法。

一类代表性工作通过改进双向搜索框架与采样引导机制来提升收敛效率，尤其关注狭窄通道等“难采样”场景下的可达性增强。Li 和 Chen（2022）提出的 ARRT-Connect 进一步依据对偶树探索得到的未勘探区域规模自适应调整抽样策略，并融合 Tahirovic 和 Ferizbegovic（2018）在 RRV 中使用的主成分分析（PCA）思想，以强化树在“细长可行走廊”内的方向性生长，从而加快狭窄环境中的探索与连通。

1	School of Mechanical Engineering, Shanghai Institute of Technology, Shanghai 201418, China

2	Corresponding Author: Xingyu Guo

Gxyhaoxianghenlihai@gmail.com

然而，上述方法总体仍以快速获得可行解为目标，缺乏对路径代价的渐进优化，因而难以保证路径最优性。同时在大尺度或存在多通道的环境中，仍可能在与最优路径无关的区域产生大量冗余采样点，影响整体效率。

为兼顾快速规划与路径质量，另一类研究从代价驱动的采样约束与结构化搜索角度对 RRT 进行改进，通过缩小有效采样域或分解搜索过程来加速收敛并提升路径质量。Zhao 等提出的 Dynamic RRT 算法创新性地引入启发式超椭球体约束采样机制，通过估计经过扩展节点的路径代价，将采样空间限制在以起点和目标点为焦点、估计路径长度为长轴的超椭球体内，大幅提升了新节点生成的有效性 [16]。同时，该算法结合帕累托支配（Pareto Dominance）原则与动态规划思想，通过选择帕累托最优节点作为新起点分解规划问题，在保证路径长度优化的前提下进一步缩短了收敛时间，其性能显著优于 RRT、RRT*及 Informed RRT等经典算法 [16]。此类方法通过采样空间约束与问题分解策略，有效平衡了收敛速度与路径质量，然而，该算法的采样约束性能仍高度依赖路径代价估计的准确性：当代价被高估时，知情子集范围过大，产生较多无效采样；当代价被低估时，知情子集可能过早收缩，削弱对可行域的覆盖与探索性，进而增加错失更优解甚至可行解的风险。此外，Dynamic RRT 采用单向搜索框架，其知情子集仅以起点和终点为焦点，在复杂环境中对全局搜索方向的引导能力有限，仍存在冗余扩展问题。

总体而言，现有改进方法在效率与最优性之间仍存在明显权衡：采样引导与双向连通策略虽可加速获得可行解，但通常缺乏渐进最优保证，而代价驱动的约束采样与问题分解方法能够提高有效采样比例并改善路径质量，但其性能对代价估计敏感，且约束域多为静态或弱自适应形式，容易在搜索早期出现过度收缩导致可达性下降，或在代价波动时引发采样比例与树扩展行为的震荡，从而削弱全局稳健性与收敛一致性。

针对上述问题，本文提出一种基于自适应非对称双椭球约束采样域的SC-RRT算法。该算法在双向并行搜索框架与中介点引导下，构造起点侧与目标侧可独立伸缩的非对称双椭球域，为克服传统约束采样对代价估计偏差的敏感性，避免采样空间过早收缩或无效膨胀，本文建立了基于搜索状态反馈的在线调节机制，其以当前最优路径代价及其历史演化为反馈，自适应调节超椭球体知情子集的尺度，在尚未获得稳定可行解阶段适度放宽约束、提升探索覆盖；当可行路径出现并持续改进时逐步强化约束、提升有效采样密度并抑制无效扩展，从而在收敛速度、路径质量与搜索稳定性之间建立可控的动态平衡。本文在不同障碍密度与空间维度的典型场景下，以 Dynamic-RRT 及经典 RRT 变体为基线开展对比实验，从首次可行解时间、路径代价、节点规模与规划稳定性等指标对上述机制的有效性进行系统评估。

- Algorithm statement

问题提出

  - 自适应非对称双椭球采样域约束策略

本文针对含随机障碍物的配置空间无碰撞路径规划，提出一种非对称双椭球域约束采样机制，实现非对称采样范围调节。兼顾全局探索与局部精细搜索，最终实现首次可行解时间（Time to First Feasible Path, TFP）、首次可行路径代价（Length of First Feasible Path, LFP）及节点规模的同步降低，成功率相对于Dynamic-RRT算法提升21%。算法整体思想如示意图 1 所示。

图1 自适应非对称双椭球采样域约束策略示意图

为构建路径规划问题的数学模型，定义工作空间W⊆Rm与构型空间C⊆Rn，其中m与n分别表示对应维度。W为机器人实际运动的笛卡尔空间，C则描述机器人所有可能的构型状态。两者在环境表示上具有等价性，均完整表达了自由区域、障碍物及环境边界信息。定义状态x∈W，将空间划分为无碰撞区域Wfree如图中白色区域与障碍区域Wobs如黑色圆形障碍物，满足W=Wfree∪Wobs。规划起点、终点分别记为xstart,xgoal∈Wfree如图1中，Π表示所有路径的集合，Πfree表示所有无碰撞路径的集合。为便于讨论，以下主要基于工作空间展开，但所有论述同样适用于构型空间。

SC-RRT算法从起点 xstart、终点 xgoal分别构建两颗随机树、并引入动态中介点 xmeet，将全局搜索解耦为 “起点—中介点”和“终点—中介点”的两个子问题，其定义见式1。其中中介点 xmeet基于双树搜索状态在线更新。与传统单一静态椭球不同，本文所构建的超椭球体并非固定形状，其参数随路径代价改进与中介点迁移更新，以xmeet为桥梁，根据搜索过程中的路径代价变化与扩展状态自适应地进行伸缩，在起点侧与目标点侧分别在构建超椭球体采样约束，保证潜在最优路径覆盖性的同时，持续压缩无效采样区域。该采样机制的核心是动态计算超椭球体的约束参数，对于任意一个方向，以起点侧为例，其定义为：

其中

- 表示从起点侧经启发式评估的最优总代价估计；

- 是从起点xstart到节点x的累积路径代价；

- 是节点x到中介点xmeet的启发式距离；

且为平衡探索与开发，引入基于搜索状态反馈的在线调节机制进行闭环调节，当路径改进停滞时主动扩大探索范围，而当改进高效时，缩小椭球并提高知情采样比例，强化局部收敛。该机制使采样策略根据搜索进程自适应调整，避免人工参数调优的局限性。

每次迭代中，检查新节点是否可与对树直接连接，否则由对树向新节点方向贪心步进逼近。通过中介点引导，两棵树的生长方向被有效约束，避免无目标扩展与搜索震荡。当双树成功连接后，对拼接路径执行节点重连优化与自适应平滑处理，生成最终可行路径。具体实现伪代码见算法1，2。

  - 算法实现1

算法1 在线调节机制: 根据当前最优路径代价 cbest及其历史序列 chist，估计改进效率并通过离散调节采样约束参数，输出超椭球扩张因子 γ与启发式采样概率 p，用于自适应控制后续迭代的“约束强度”和“信息采样占比”

  - 算法实现2

算法2 自适应非对称双椭球采样域约束（Adaptive Dual-Ellipsoid Constrained Sampling）: 以双向树 TA,TB及会合点 xmeet为基础，将全局规划分解为“起点–会合点”和“会合点–终点”两个子问题，分别构造非对称双椭球域并自适应更新其尺度，随后在选定超椭球内进行均匀采样并通过可行性验证得到候选采样点 xsample

图2 SC-RRT算法总体框架。算法由搜索规划主循环、基于搜索状态反馈的在线调节机制、自适应双超椭球约束采样和可行性验证与双树连接四个模块组成。虚线箭头表示从搜索状态到采样参数调节的反馈路径。

Fig. 2 Overall framework of the SC-RRT algorithm. The algorithm consists of four modules: the search planning main loop, the search-state-based online adjustment mechanism, the adaptive dual-ellipsoid constrained sampling, and the feasibility verification with bidirectional tree connection. Dashed arrows indicate the feedback path from search state to sampling parameter regulation.

### 图3：2D环境下路径规划可视化对比（环境配置见表1；每幅图显示：障碍物、两棵树、最终路径、超椭球边界；）

- Simulation

为系统评估 SC-RRT 在环境规模与障碍物密度保持一致条件下的性能，本研究在不同维度下开展了一系列仿真实验（见表2）。评估指标包括收敛时间与路径长度：其中，收敛时间定义为算法获得首个可行解所需的计算时长；路径长度视为路径代价的度量，即从起始状态到目标状态的累计代价。

Table 1 Environment configurations

| Dimension | Environment | Obstacles | Start | Goal |
| --- | --- | --- | --- | --- |
| 2 | 1500*1500 | 225 | (75, 75) | (1425, 1425) |
| 3 | 1500*1500*1500 | 400 | (75, 75, 75) | (1425, 1425, 1425) |

据对路径规划的过往研究中，选取了几个相对前沿的路径规划算法进行对比，验证SC-RRT的性能和稳定性，为确保不同路径规划算法性能对比的公平性，本文在相同的环境空间和障碍物配置下，对多种算法的路径长度、收敛时间等性能指标进行评估，以表1中规定的环境为例，在限定最大迭代次数五千次的情况下进行对比，进行500次运行。

通过对SC-RRT与Dynamic-RRT算法的性能进行对比。从表3可以看出，Dynamic-RRT算法在得到更多时间支持的情况下其渐进最优性才能逐步发挥，但大多数情况下对路径规划的实时性要求更高，故需要在此做出相应取舍，并且当随机节点xrand选择的概率偏向目标节点时，它们的两个指标都有所提高。此外，可以明显看出，在保持相同路径长度的情况下，SC-RRT凭借在线调节机制（Search State Feedback-Based Online Regulation）生成的路径收敛时间及采样次数远优于Dynamic-RRT算法。实验证明SC-RRT在平衡收敛时间和路径长度方面具有明显的优势。从表4可以观察到，Dynamic-RRT算法需要更多的时间和迭代次数才能达到与SC-RRT算法相同的路径长度。虽然在运行时间没有限制的情况下，Dynamic-RRT可以获得更短的路径，但由于收敛时间的极速增加，这并不显著有用。

图9 不同维度和不同区间的路径规划结果，黑色球型物体代表均匀分布的障碍物，起始节点设置如表1所示，图a-d和e-h显示了SC-RRT在2维和3维的性能，PID =（0.45，0.07，0.18）

Table 2 All Algorithm Comparison

| Algorithm | Path Length /mm | Planning Time /s | Smoothness | Path Efficiency |
| --- | --- | --- | --- | --- |
| RRT | 2583.74 | 0.054 | 0.4919 | 0.8217 |
| RRT-connect | 2216.33 | 0.029 | 0.4487 | 0.9579 |
| RRT* | 2241.20 | 15.843 | 0.3225 | 0.9495 |
| Dynamic-RRT | 2440.14 | 0.111 | 0.4639 | 0.8696 |
| SC-RRT | 2132.33 | 0.071 | 0.0883 | 0.9956 |

此外，文献[]同时证明了其所生成的路径长度均比RRT路径短，规划时间比RRT*更少，如表5所示，SC-RRT布局在生成的路径节点数量上优于其生成路径，且路径长度更短，规划时间更少，也就是说SC-RRT算法性能更优于某某算法

- Conclusion

针对采样式路径规划算法在障碍密集环境中有效采样比例低、收敛速度慢且知情子集对代价估计偏差敏感的问题，本文提出了SC-RRT算法。该算法包含两个紧密耦合的核心机制

一，自适应非对称双椭球约束采样（ADCS）。引入基于双树搜索状态在线更新的动态中介点，将全局规划任务分解为"起点—中介点"与"终点—中介点"两个方向性子问题，并分别构造尺度可独立调节的超椭球知情子集。与Dynamic-RRT以起点和终点为固定焦点的单一对称椭球不同，ADCS的非对称双椭球结构能够针对两侧搜索进展差异进行方向性压缩与扩展，在保持探索覆盖性的同时显著提高潜在可行通道上的采样密度，有效缓解了引言中指出的单向框架全局引导能力不足的问题。

二，基于搜索状态反馈的在线调节机制（SSFOR）。以当前最优路径代价及其近期历史序列为反馈信号，实时输出超椭球扩张因子γ与知情采样概率p，对知情子集的体积与采样分配比例实施闭环调节。当路径改进停滞时自动扩大探索范围，当改进高效时逐步收紧约束以强化局部收敛，从而克服传统方法中代价高估导致采样域过大或代价低估导致采样域过早收缩的敏感性问题。

在二维与三维不同障碍密度的典型场景中，以相同环境配置与计算预算为约束，SC-RRT对RRT、RRT-Connect、RRT*、Dynamic-RRT四种基线算法进行了系统对比。实验结果表明，SC-RRT在路径质量、规划效率与路径平滑性三个维度上均取得了综合最优表现。具体而言：在路径质量方面，SC-RRT的平均路径长度为2132.33 mm，较Dynamic-RRT缩短约12.6%，较RRT*缩短约4.9%，路径效率（直线距离与实际路径长度之比）达到0.9956，在所有对比算法中最接近理论最优值；在规划效率方面，SC-RRT的平均规划时间为0.071 s，与RRT（0.054 s）、RRT-Connect（0.029 s）处于同一数量级，同时比RRT*（15.843 s）快逾两个数量级；在路径平滑性方面，SC-RRT的平滑度指标（转角序列标准差）为0.0883 rad，较Dynamic-RRT（0.4639 rad）降低81.0%，较RRT*（0.3225 rad）降低72.6%，平均转角仅为5.49，远低于RRT（34.38）和Dynamic-RRT（34.01）。此外，SC-RRT路径的平均安全间隙达56.92 mm，体现了ADCS机制在缩小采样域的同时对障碍物间距的隐式保持能力。在ADCS的双向非对称约束与SSFOR闭环调节的协同作用下，SC-RRT在不同维度空间与障碍分布条件下均表现出更一致的收敛行为、更少的冗余扩展与更高的规划稳定性。

尽管如此，当前工作仍存在一定局限性。当前代价函数仅优化欧氏路径长度，未直接考虑曲率约束或能量消耗等运动学指标；此外，路径后处理平滑模块引入的非线性变形在个别情况下可能增加路径长度，其与上游规划质量的耦合效应尚未进行完整的理论刻画。

Authors' Contributions All authors contributed to the study conception and design. Penglei Zhao wrote the manuscript and did the research. Tech- nical support was provided by Yinghui Chang, Weikang Wu, Hongyin Luo, Zhixin Zhou. Valuable comments on manuscript revisions were put forward by Yanping Qiao, Ying Li, Chenhui Zhao, Zenan Huang, Bijing Liu, Xiaojie Liu, Shan He. Prof. Donghui Guo provided manuscript writ- ing guidance. All authors read and approved the final manuscript.

Funding This work is supported by the Key project's funding of NSFC (No.61836010).

Code availability The source code in this paper will be disclosed upon acceptance for publication of this paper. Before accepting, please E-mail to 2,399,496,491@qq.com if the source code is needed.

Declarations

Ethics approval Not applicable.

Competing interests The authors have no relevant financial or non- financial interests to disclose.

Consent to participate Not applicable.

Consent for publication Not applicable.

References

- Ji, S., et al.: Learning-Based Automation of Robotic Assembly for Smart Manufacturing. Proc. of the IEEE. 109, 423–440 (2021)

- Perzylo, A., et al.: SMErobotics Smart Robots for Flexible Manu- facturing. IEEE ROBOT AUTOM MAG. 26, 78–90 (2019)

- Hvilshoj, M., et al.: Autonomous industrial mobile manipula- tion (AIMM): past, present and future. Industrial Robot-the International Journal of Robotics Research and Application. 39, 120–135 (2012)

- Roa, M.A., Berenson, D., Huang, W.: Mobile Manipulation: Toward Smart Manufacturing. IEEE Robot. Autom. Mag. 22, 14–15 (2015)

- Gonzalez, A.G.C., et al.: Supervisory Control-Based Navigation Architecture: A New Framework for Autonomous Robots in Industry 4.0 Environments. IEEE Trans. Indust. Inf. 14, 1732–

1743 (2018)

- Pan, C., et al.: A Novel Algorithm for Wafer Sojourn Time Anal- ysis of Single-Arm Cluster Tools With Wafer Residency Time Constraints and Activity Time Variation. IEEE Transactions on Systems Man Cybernetics-Systems. 45, 805–818 (2015)

- Li, Z., et al.: A Fault-Tolerant Method for Motion Planning of Industrial Redundant Manipulator. IEEE Trans. Industr. Inf. 16, 7469–7478 (2020)

- Baumann, D., et al.: Wireless Control for Smart Manufactur- ing: Recent Approaches and Open Challenges. Proc. IEEE 109, 441–467 (2021)

- Li, S., Han, K., Li, X., et al.: Hybrid Trajectory Replanning- Based Dynamic Obstacle Avoidance for Physical Human-Robot Interaction. J Intell Robot Syst 103, 41 (2021)

- Hart, P.E., Nilsson, N.J., Raphael, B.: A Formal Basis for the Heuristic Determination of Minimum Cost Paths. IEEE Trans- actions on Systems Science and Cybernetics. 4, 100–107 (1968)

- Karaman, S. and E. Frazzoli.: Incremental Sampling-based Algorithms for Optimal Motion Planning. in Robotics: Sci. Syst. 2010. (2010). https://doi.org/10.48550/arXiv.1005.0416

- Wang, J.K., et al.: Neural RRT*: Learning-Based Optimal Path Planning. IEEE Trans. Autom. Sci. Eng. 17, 1748–1758 (2020)

- Li, Y., et al.: Neural Network Approximation Based Near-Opti- mal Motion Planning With Kinodynamic Constraints Using RRT. IEEE Trans. Industr. Electron. 65, 8718–8729 (2018)

- Lavalle, S.M.: Rapidly-exploring random trees : A new tool for path planning. Research Report. https://www.researchgate.net/ publication/2639200_Rapidly-Exploring_Random_Trees_A_ New_Tool_for_Path_Planning. (1998)

- Kavraki, L.E., et al.: Probabilistic roadmaps for path planning in high-dimensional configuration spaces. IEEE Trans. Robot. Autom. 12, 566–580 (1996)

- Wang, J.K., Meng, M.Q.H., Khatib, O.: EB-RRT: Optimal Motion Planning for Mobile Robots. IEEE Trans. Autom. Sci. Eng. 17, 2063–2073 (2020)

- Kusuma, M., Riyanto, and C. Machbub.: Humanoid Robot Path Planning and Rerouting Using A-Star Search Algorithm. 2019 IEEE International Conference on Signals and Systems (Icsig- sys). (2019). https://doi.org/10.1109/ICSIGSYS.2019.8811093

- An, B., Kim, J., Park, F.C.: An Adaptive Stepsize RRT Planning Algorithm for Open-Chain Robots. IEEE Robotics and Automa- tion Letters. 3, 312–319 (2018)

- Wang, W., Zuo, L., Xu, X.: A Learning-based Multi-RRT Approach for Robot Path Planning in Narrow Passages. J. Intell. Rob. Syst. 90, 81–100 (2018)

- Aguinaga, I., Borro, D., Matey, L.: Parallel RRT-based path plan- ning for selective disassembly planning. Int. J. Adv. Manuf. Tech- nol. 36, 1221–1233 (2008)

- Bruce, J. and M.M. Veloso.: Real-time randomized path planning for robot navigation. Robocup 2002: Robot Soccer World Cup Vi. 2752, 288–295 (2003)

- Jr, J. and S.M. Lavalle.: RRT-Connect: An Efficient Approach to Single-Query Path Planning. in Proceedings of the 2000 IEEE International Conference on Robotics and Automation, ICRA 2000, April 24–28, 2000, San Francisco, CA, USA. (2000).

- Moon, C.B., Chung, W.: Kinodynamic Planner Dual-Tree RRT (DT- RRT) for Two-Wheeled Mobile Robots Using the Rapidly Exploring Random Tree. IEEE Trans. Industr. Electron. 62, 1080–1090 (2015)

- Karaman, S., Frazzoli, E.: Sampling-based algorithms for optimal motion planning. Int. J. Robot. Res. 30, 846–894 (2011)

- Chen, L., et al.: A Fast and Efficient Double-Tree RRT*-Like Sampling-Based Planner Applying on Mobile Robotic Systems. IEEE-Asme Transactions on Mechatronics. 23, 2568–2578 (2018)

- Gammell, J.D., S.S. Srinivasa, and T.D. Barfoot.: Informed RRT*: Optimal Sampling-based Path Planning Focused via Direct Sam- pling of an Admissible Ellipsoidal Heuristic. 2014 IEEE/Rsj International Conference on Intelligent Robots and Systems (Iros 2014). 2997–3004 (2014)

- Salzman, O., Halperin, D.: Asymptotically Near-Optimal RRT for Fast, High-Quality Motion Planning. IEEE Trans. Rob. 32, 473–483 (2016)

- Qi, J., Yang, H., Sun, H.X.: MOD-RRT*: A Sampling-Based Algorithm for Robot Path Planning in Dynamic Environment. IEEE Trans. Industr. Electron. 68, 7244–7251 (2021)

- Gammell, J.D., S.S. Srinivasa, and T.D. Barfoot.: On Recursive Random Prolate Hyperspheroids. Eprint Arxiv, (2014). https:// doi.org/10.48550/arXiv.1403.7664

- Sun, H.Y., Farooq, M.: Note on the generation of random points uniformly distributed in hyper-ellipsoids. Proceedings of the Fifth International Conference on Information Fusion I, 489–496 (2002)

- Gammell, J.D. and T.D. Barfoot.: The Probability Density Func- tion of a Transformation-based Hyperellipsoid Sampling Tech- nique. Stat., (2014). https://doi.org/10.48550/arXiv.1404.1347

- De Ruiter, A.H.J., Forbe, J.R.: On the Solution ofWahba’s Prob- lem on S O (n). Journal of the Astronautical Sciences. 60, 1–31 (2014)

- Guo, G., et al.: Predicting Pareto Dominance in Multi-objec- tive Optimization Using Pattern Recognition. in Second Inter- national Conference on Intelligent System Des. Eng. Appl. (2012). https://doi.org/10.1109/ISdea.2012.589

35  Sanders and Robert: The Pareto Principle: its Use and Abuse.

J. Serv. Mark. 1(37), 40 (1987)

Publisher's Note Springer Nature remains neutral with regard to jurisdictional claims in published maps and institutional affiliations.

Springer Nature or its licensor (e.g. a society or other partner) holds exclusive rights to this article under a publishing agreement with the author(s) or other rightsholder(s); author self-archiving of the accepted manuscript version of this article is solely governed by the terms of such publishing agreement and applicable law.

Penglei Zhao received the B.S. degree in Electronics and informa- tion engineering from Xiamen University, China, in 2019. He is cur- rently pursuing the M.S. degree in the School of Electronic Science and Engineering at Xiamen University. His research interests include path planning, robotic arm control and artificial intelligence.

Yinghui Chang received the M.S. degree and now is a senior engineer. He is mainly engaged in the design and application development of digital SoC integrated circuits. At the same time, he is engaged in the research of reasoning AI chip design and application technology.

Weikang Wu received the B.S. degree in microelectronics from Xidian University in 2011, and the Ph.D. degree in electronics engineering and computer science from Peking University in 2016. Her current inter- ests include computer vision and VLSI radiation-hardened-by-design techniques.

Hongyin Luo Graduated from Xiamen University in 2014 with a doctorate. He once worked in Newland Computer Co., Ltd. and served as the director of Xiamen University-Newland SOC joint laboratory. He was rated as a high-level introduced talent in Fuzhou. In 2018, he founded Xiamen Embedeep Technology Co., Ltd. as the founder and CEO. At present, he is mainly engaged in artificial intel- ligence, network on chip and integrated circuit design. Since 2013, He has participated in or presided over the research and development of 7 chips, participated in a number of national or provincial or municipal major scientific research projects, published more than 10 papers in domestic and foreign journals, led the research of EEP-TPU IP core technology, and has more than 10 invention patents for core technologies related to artificial intelligence processors.

Zhixin Zhou received the B.S. and M.S. degrees from Xiamen Uni- versity. He once worked in Xiamen University-Newland SOC joint laboratory and is now the co-founder and hardware technology director of Xiamen Embedeep Technology Co., Ltd. He has participated in the research of 7 chips and has rich practical experience in chip develop- ment. He is currently leading the FPGA development and ASIC / SOC chip development of the EEP-TPU IP.

Yanping Qiao received the B.S. and M.S. degrees in Electrical engi- neering and automation from Guilin University of Electronic Technol- ogy, China, in 2013 and 2016, respectively. She is currently pursuing the Ph.D. degree in the School of Electronic Science and Engineering with Xiamen University. Her research interests include neural net- works, integrated circuit optimization design and artificial intelligence.

Ying Li received the Bachelor degree from JiangNan University, Wuxi, China, in 2019. She is currently pursuing the Master degree in Xia- men University, Xiamen, China. Her research directions mainly include digital integrated circuit design and evolvable hardware.

Chenhui Zhao received the Bachelor and Master degrees from Shan- dong University of Science and Technology, Qingdao, China, in 2015 and 2018, respectively. She is currently pursuing the Ph.D. degree in the School of Electronic Science and Engineering at Xiamen Univer- sity, Xiamen, China. Her research directions mainly include neural networks, evolutionary computation and artificial intelligence.

Zenan Huang received the B.S. degrees in School of Information Sci- ence and Technology from Xiamen University, China, in 2016. He is currently pursuing the Ph.D. degree in the School of Electronic Science and Engineering with Xiamen University. His research interests include artificial intelligence and neural network.

Bijing Liu received the Bachelor degree from Hainan University, Hainan, China, in 2016, and graduated with a Master’s degree from Xiamen University, Xiamen, China, in 2020. He is currently pursuing the Ph.D. degree in Xiamen University, Xiamen, China. His research directions mainly include artificial intelligence and image processing.

Xiaojie Liu received the B.S. degree in Applied Math from University of California San Diego, USA, in 2014, and the M.S. degree in Electrical and Computer Engineering from Southern Illinois University-Carbondale, USA, in 2019, respectively. She is currently an Assistant Engineer of the School of Electronic Science and Engineering at Xiamen University. Her research areas include artificial intelligence, data analysis, and statistics.

Shan He received the B.S. degree in communication engineering, the M.S. degree in the IC design and verification from Hefei University of Technol- ogy, China, in 2004 and 2007, respectively. In 2007, she joined Xiamen University as an Assistant Engineer. She is currently a Senior Engineer of the School of Electronic Science and Engineering. Her research areas include computer networking, artificial intelligence and IC design.

Donghui Guo received the B.S. degree in radio physics, the M.S. and the Ph.D. degrees in semiconductor from Xiamen University, China, in 1988, 1991, and 1994, respectively. In 1994, he joined Xiamen Uni- versity as a Faculty Member, where he has been a Full Professor, since 2002. He held a post-doctoral, a research fellow, a senior scientist, and a visiting scholar position, respectively, with the City of Hong Kong, Uni- versity of Ulster, Lawrence Berkeley Laboratory, University California at Berkeley, and University of Illinois at Urbana-Champaign, and served as the Vice-Dean with the School of Information Science and Technol- ogy, Xiamen University, from 2008 to 2012. He is currently a senior member of IEEE and the Director of IC Design & IT Research Center. His research areas include computer networking, artificial intelligence, optimization computing, IC design, nano device, and BioMEMS.

1\PID是"搜索动态调节器"，不是"路径优化器"

2\由于平滑模块具有较强的非线性特性，后处理后的渐近路径长度未被采用作为 PID 参数整定的主要评价指标。

3\ P 增大 → 控制算法响应更快，树扩展更“激进”，可能更快找到路径。但 P 过大 → 节点扩展方向可能不够稳定，导致平滑后长度增加。

4\ I 值较小 (0.01)积分作用有限，对整体路径收敛速度影响小。

5\  D 值 0.08,对树节点预测方向微调，帮助路径平滑，但不改变主要收敛速度。

| 指标 | 说明 | 预期稳定性 |

|------|------|------------|

| **IFP** | Iterations to First Feasible Path | std ≈ 3-40 |

| **LFP** | Length of First Feasible Path | std ≈ 6-40 |

| **TFP** | Time to First Feasible Path | std ≈ 0.1-9 |

PID参数对比实验中观察到的首次求解时间缩短完全归因于基于PID的采样偏置调节，独立于任何轨迹后处理。

- 创建真正的PID自适应采样函数 samplePIDAdaptive.m

  - P项: 根据当前距离误差控制目标偏置强度

  - I项: 累积历史误差，避免局部陷阱

  - D项: 抑制采样震荡，控制扰动幅度

- PID 参数对首次可行路径性能的影响  由于平滑模块具有较强的非线性特性且在部分情况下可能失效，后处理后的渐近路径长度未被采用为 PID 参数调优的主要评价指标。相反，性能评估侧重于首次可行路径的特性，因为这更能直接反映 PID 对搜索动力学的影响。(免责声明)如图9所示，当SC-RRT算法运行的时候，不管是在三维亦或者是在二维空间中，其生成的路径长度都随着PID在线学习框架的调控下采样效率更高，冗余产生的节点数量更少。此外，随着知情子集约束下的采样范围更精确，规划过程的收敛时间也随之减少，如表2,但并不在PID中的优化控制中，并不是随意调节的，当参数区间超出某一特定接线时，并不能获得较好的路径，如图某，与图某和图某相比，路径开始变得不平滑。此外在表2中可以看到，与其他情况相比，这些情况下所消耗的时间有所增加，因为当区间取过小的值，如0.5时，表现出搜索节点大幅度增多的现象，使得路径节点仅在这两个节点之间随机选择，导致无法充分利用非对称双椭球采样域进行约束采样路径效果。而当区间变大时，也会出现同样的问题。图8d和h说明了PID值为0.7时的模拟结果，从中我们可以再次观察到收敛时间和路径成本增加的现象，就像在间隔过小的情况下一样。当区间趋于无穷大时，帕累托占优机制甚至可能完全失效，导致动态RRT退化为直接从知情子集采样的RRT。 相应地，区间需要在合理的范围内选取。

- 不同参数间的影响    为了更准确直观的控制不同的比例积分微分参数的值，我们在（0，200）范围内模拟了不同区间对路径成本和收敛时间的影响。结果见图10。从图10a中，一方面，我们可以看到当P值积分项超过80的上界时，收敛时间几乎没有上升，这表明动态RRT已经退化为从知情子集直接采样的RRT，尽管路径成本仍然随机波动。另一方面，我们可以观察到当I值微分项区间小于4的下界时，收敛时间开始上升，而路径成本仍然保持下降的趋势。这意味着通过继续减小4以下的区间，我们仍然可以获得更好的路径，尽管图8a和e中描述的情况可能会出现。 此外，在D值微分项在某某区间内时，路径开始趋于平稳。在这些上下界之间，收敛时间和路径成本均保持下降趋势。因此，我们可以通过在这个范围内选择合适的区间来实现收敛时间和路径长度之间的平衡。在此条件下，P=4,I=8,D=12将导致收敛时间为0 . 034 s，路径成本为1157 . 4 .如果需要更短的路径，区间可以设置为2或3，代价是收敛时间的显著增加。

### 本文的目的是生成一条平衡收敛时间和路径长度的路径，从实验所得到的结果看，可以做出如下观察：

- PID性能分析实验结果表明，PID 控制器对 SC-RRT 的收敛动力学具有显著影响，尤其体现在首次可行解时间和搜索稳定性方面。适中的比例增益配合中等强度的积分项，在快速收敛与稳定采样行为之间实现了最佳平衡。

- 与传统RRT变体的性能对比仿真表明，其收敛时间短于RRT，且其生成的路径长度更优于RRT-Star,此外，于文献【】算法相比，SC-RRT在平衡收敛时间和路径长度等方面表现的更好，以至于能够在相对较短的时间内获得同时满足更短的路径长度，更少的规划时间，更少的路径节点以及更平滑的路线，而Informed-rrt算法则需要去花费更多路径点乃至更多的时间去计算一条等价的路径。

- 稳定性对比仿真说明，在不同的环境和不同起始节点和目标节点距离下， SC-RRT算法所规划的最终路径比之上述路径规划算法及其变体更加稳定，此其生成的路径更加集中一致。